Assignment 2

Experiment 1: ICMP Packet Analysis

Aim:

To capture and analyze ICMP packets (ping request and reply).

Procedure:

- Open Wireshark

- Start packet capture

- Run command:

ping google.com

- Stop capture

- Apply filter: icmp

Observation:

- Protocol: ICMP

- Source IP: 192.168.29.1

- Destination IP: 192.168.29.212

- Message: Echo Request

- TTL: 64

Result:

ICMP packets were successfully captured and analyzed.

Experiment 2: TCP Three-Way Handshake

Aim:

To analyze TCP packets and observe the TCP three-way handshake.

Procedure:

- Open Wireshark and start packet capture on the active interface (Wi-Fi).

- Open a web browser and visit any website.

- Stop the capture after a few seconds.

- Apply the display filter: tcp

- SYN

  - 172.16.75.158 → 54.147.20.226

  - [SYN]

  - Client is requesting connection

- SYN-ACK

  - 54.147.20.226 → 172.16.75.158

  - [SYN, ACK]

  - Server acknowledges and responds

- ACK

  - 172.16.75.158 → 54.147.20.226

  - [ACK]

  - Client confirms → connection established

Experiment 3: UDP Packet Analysis

Aim:

To capture and analyze UDP packets.

Procedure:

- Open Wireshark and start packet capture on Wi-Fi

- Allow system/network activity to generate traffic

- Stop capture after a few seconds

- Apply the display filter: udp

Observation:

From the captured packet:

- Protocol: UDP (mDNS)

- IP Version: IPv6

- Source Address: fe80::5d77:4bf2:627b:6737

- Destination Address: ff02::fb (Multicast address)

- Source Port: 55779

- Destination Port: 5353

- Packet Length: 98 bytes

- Query Type: Standard query (PTR record)

- Service Queried: _asquic._udp.local

Result:

UDP packets were successfully captured.

Experiment 4: DNS Packet Analysis

Aim:

To capture and analyze DNS query and response packets.

Procedure:

- Open Wireshark and start packet capture on Wi-Fi

- Open a web browser and visit any website

- Stop capture after a few seconds

- Apply the display filter: dns

Observation:

From the captured packets:

- Protocol: DNS (over UDP)

- Source IP Address: 172.16.75.158

- Destination IP Address: 172.31.1.6 (DNS server)

- Source Port: 51062

- Destination Port: 53

- Packet Length: 81 bytes

- Domain Queried: platform.linkedin.com

- Query Type: A record (IPv4 address resolution)

- Packet Type: Standard DNS query

Experiment 5: HTTP Packet Analysis

Aim:

To capture and analyze HTTP packets.

Procedure:

- Open Wireshark and start packet capture on Wi-Fi

- Open a web browser and access a non-secure website (HTTP)

- Stop capture after a few seconds

- Apply the display filter: http

Observation:

From the captured packets:

- Protocol: HTTP

- Source IP Address: 172.16.75.158

- Destination IP Address: 103.195.184.203 (from selected packet)

- Source Port: 62935

- Destination Port: 80

- Request Method: GET

- HTTP Version: HTTP/1.1

- Response Code: 304 Not Modified

- Data Visibility: Plain text (readable request and headers)

Result:

HTTP packets were successfully analyzed.

Experiment 6: ARP Packet Analysis

Aim:

To analyze ARP request and reply packets.

Procedure:

- Open Wireshark and start packet capture on the active interface (Wi-Fi)

- Generate ARP traffic by:

  - Reconnecting to the network or

  - Running command: arp -d *

- Stop capture after a few seconds

- Apply the display filter: arp

- Protocol: ARP

- Packet Type: ARP Request

- Source MAC Address: 74:8e:f8:90:80:78

- Destination MAC Address: ff:ff:ff:ff:ff:ff (Broadcast)

- Query:

  - “Who has 172.16.94.53? Tell 172.16.64.1”

- Packet Length: 60 bytes

Key observations:

- ARP request is sent as a broadcast to all devices

- The sender is trying to find the MAC address of the target IP

- Broadcast address confirms all devices receive the request

Experiment 7: MAC & IP Address Identification

Aim:

To identify source and destination MAC and IP addresses from captured packets.

Procedure:

- Open Wireshark and select any captured packet

- Expand the Ethernet II section to view MAC addresses

- Expand the Internet Protocol (IP) section to view IP addresses

- Note down the source and destination details

Observation:

- Source IP: 172.16.75.158

- Destination IP: 54.147.166.142

- Source MAC: (from Wireshark)

- Destination MAC: (from Wireshark)

Result:

MAC and IP addresses were successfully identified.